COSHH Information

Host Odour Neutraliser

1.  PRODUCT AND COMPANY IDENTIFICATION

Product name:  HOST® SMOKE-X® Odour Eliminator

Product category:  general purpose, non-aerosol deodorant

Product usage: spray on a fine mist to deodorise interior furnishings and surfaces (refuse containers, kennels) nursing homes, hotels, residential and commercial premises.

Manufacturer:	Racine Industries, Inc.

1405 16th Street

Racine, WI 53403 USA

Tel:  00 1-262-637-4491

Emergency contact after hours:  G. J. Botting – 00 1-262-681-7431

UK Head Office:	Host

Unit 6b Capenhurst Technology Park

Capenhurst Lane

Capenhurst

Cheshire

CH1 6EH

Tel: 0151 347 1900

Fax: 0151 347 1901

Email host@hostvs.co.uk

www.hostvonschrader.co.uk

2.  COMPOSITION / INGREDIENTS

Hazardous ingredient:

Ethyl Alcohol			Cas #64-17-5		1.38%		TLV = 1,000 ppm

Note:  dilute product 1:16 for use.  Use dilution contains 0.86% alcohol.

3.  HAZARDS IDENTIFICATION

This product is not classified as hazardous to health or the environment.

4.  FIRST-AID MEASURES

In case of overexposure or extreme sensitivity:

Skin contact:  rinse with water

Inhalation:  remove to fresh air

Eye contact:  rinse with water

Ingestion:  rinse mouth with water; do not induce vomiting; seek medical advice if large amount has been swallowed

5.  FIRE-FIGHTING MEASURES

The product is not flammable

6.  ACCIDENTAL RELEASE MEASURES.

Mop or soak up with absorbent material such as cotton or paper toweling.  No special precautions 	necessary.  Rinse area with water and mop dry.

7.  HANDLING AND STORAGE

Does not require any special handling.  Recommended temperature for storage is 32 to 100°F

(0-38°C)

8.  PERSONAL PROTECTION / EXPOSURE CONTROLS

No protective equipment is necessary.

Gloves advised for sensitive skin; ventilation advised for asthmatics.

No known harmful effect from repeated, long-term exposure.

9.  PHYSICAL AND CHEMICAL PROPERTIES

Physical state:  liquid

Colour:  concentrate - amber, cloudy; use dilution (1:16) - colourless, cloudy

Odour:  pine

pH:  concentrate - 8.5; use dilution (1:16) - 7.5

Specific gravity: 1.01

Vapour pressure:  (mm Hg):  18.41 @ 70°F (21°C)

Boiling point:  >197°F (>98°C)

Flash point: > 212°F (>100°C) by Setaflash cc

10.  STABILITY AND REACTIVITY

Stable

11.  TOXICOLOGICAL INFORMATION

No toxic effects			TEST PERFORMED			RESULT

Not a skin irritant			OECD 404 Primary Skin Irritation	Negative

No oral toxicity			OECD 401 - Oral Toxicity		LD50 > 5 gm/kg

No inhalation toxicity			OECD 403 - Inhalation Toxicity	LC50  > 5.0 mg/l

Not an eye irritant			OECD 405 - Eye Irritation		Negative:  use dilution

Slight: concentrate

12.  ECOLOGICAL INFORMATION

Biodegradable

13.  DISPOSAL CONSIDERATIONS

No special precautions                 According to local regulations - can be landfilled

14.  TRANSPORT INFORMATION

No special labeling required.

15.  REGULATORY INFORMATION

None.  Not a regulated product.

DOT (Canada & U.S.):  no special labeling.

Canadian WHMIS:  not a controlled substance.  No special labeling.

Canadian Consumer Regulations:  not a restricted substance.  No special labeling.

California Prop. 65:  no listed ingredients.  No special labeling.

New Jersey Right-to-Know:  identical to consumer goods.  No special labeling.

EEC Regulations:  no special labeling.  Not a dangerous preparation.

16.  OTHER INFORMATION

HMIS (Hazardous Materials Information System) rating:  Health = 0;

Flammability = 0; Reactivity = 0; Personal Protection = none required.

Scale:  0 = minimal; 1 = slight; 2 = moderate; 3 = high; 4 = extreme